  - A ∗ (!A + B ∗ B) + !(B + A) ∗ (!A + B)

    - A*(!A+B*B)+!B*!A(!A+B)  Using Demorgan’s Theorem

    - A(!A+B)+!B*!A(!A+B)  Idempotent Law

    - (!A+B)(!B*!A+A)  Distributive Law

    - (!A+B)(!B+A)  Absorption Law

    - (!B+A)!A+(!B+A)B  Distribution

    - !A*!B+!A*A+(!B+A)*B  Distribution

    - !A*!B+0+(!B+A)*B  Complement Law

    - !A*!B + B*A  Identity Law

Answer: !A!B + BA

  - !(C ∗ B) + (A ∗ B ∗ C) + !(A + C + !B)

    - !C*!B+A*B*C+!(A+C+!B)  Demorgan Theorem

    - !C*!B + A*B*C + !A*!C*!!B  Demorgan Theorem

    - !C*!B+A*B*C+!A*!C*B  Involution Law

    - A*B*C + !C*(!A*B+!B)  Distributive Law

    - A*B*C+!C(!A+!B)  Absorption Law

    - A*B*C+!C*!A+!C*!B  Distribution

Answer: ABC+!C!A+!C!B

  - (A + B) ∗ (!A + C) ∗ (!C + B)

    - (!A+C)(!C+B)*A+(!A+C)(!C+B)*B  Distribution

    - (!C+B)*A*!A+(!C+B)*A*C+(!A+C)*(!C+B)*B  Distribution

    - 0+(!C+B)*A*C+(!A+C)*(!C+B)*B  Complement Law

    - (!C+B)*A*C+(!A+C)*(!C+B)*B  Identity Law

    - A*C*!C+A*C*B+(!A+C)*(!C+B)*B  Distribution

    - 0+A*C*B+(!A+C)*(!C+B)*B  Complement Law

    - A*C*B + (!A+C)*(!C+B)*B  Identity Law

    - A*C*B+(!C+B)*B*!A+(!C+B)*B*C  Distribution

    - A*C*B+B*!A*!C+B*!A+(!C+B)*B*C  Distribution

    - A*C*B + B*!A*!C+B*!A+(!C + B) B*C  Idempotent Law

    - A*C*B + B*!A + (!C+B)B*C  Absorption Law

    - B*(A*C+!A)+(!C+B)*B*C  Distributive Law

    - B(C+!A)+(!C+B)*B*C  Absorption Law

    - B*C+B*!A+(!C+B)*B*C  Distribution

Answer: BC+B!A

B*C+B*!A  Absorption Law

  - (!A + C) ∗ (!B + D + A) ∗ (D + A ∗ !C) ∗ (!D + A) = 1

    - A(!B+D+A)(D+A!C)(!D+A)!A+A(!B+D+A)(D+A!C)(!D+A)C  Distribution

    - 0+A(!B+D+A)(D+A!C)(!D+A)C  Complement Law

    - A(!B+D+A)(D+A!C)(!D+A)C  Identity Law

    - A(D+A!C)(!D+A)C!B+A(D+A!C)(!D+A)CD+A(D+A!C)(!D+A)CA  Distribution

    - A(D+A!C)(!D+A)C!B+A(D+A!C)(!D+A)CD+A(D+A!C)(!D+A)C  Idempotent Law

    - A(D+A!C)(!D+A)CD+A(D+A!C)(!D+A)C  Absorption Law

    - A(D+A!C)(!D+A)C  Absorption Law

    - A(!D+A)CD+A(!D+A)CA!C  Distribution

    - A(!D+A)CD+A(!D+A)C!C  Idempotent Law

    - A(!D+A)CD+0  Complement Law

    - A(!D+A)CD  Identity Law

    - ACD!D+ACDA  Distribution

    - ACD!D+ACD  Idempotent Law

    - 0+ACD  Complement Law

    - Simplifies down to C * D * A = 1

Therefore, there is only 1 solution:

C = 1, D = 1, and A = 1

  - (((!K ∗ L ∗ N) ∗ (L + M)) + ((!K + L + N) ∗ (K ∗ !L ∗ !M))) ∗ (!K + !N) = 1

    - (!K+!N)!KLN(L+M)+(!K+!N)(!K+L+N)K!L!M  Distribution

    - !KLN(L+M)!K+!KLN(L+M)!N+(!K +!N)(!K+L+N)K!L!M  Distribution

    - !KLN(L+M)+!KLN(L+M)!N+(!K +!N)(!K+L+N)K!L!M  Idempotent Law

    - !KLN(L+M)+0+(!K+!N)(!K+L+N)K!L!M  Complement Law

    - !KLN(L+M)+(!K+!N)(!K+L+N)K!L!M  Identity Law

    - !KLNL + !KLNM+ (!K+!N)(!K+L+N)K!L!M  Distribution

    - !KLN + !KLNM + (!K+!N)(!K+L+N)K!L!M  Idempotent Law

    - !KLN + (!K+!N)(!K+L+N)K!L!M  Absorption Law

    - !KLN + (!K+L+N)K!L!M!K+(!K+L+N)K!L!M!N  Distribution

    - !KLN+0+(!K+L+N)K!L!M!N  Complement Law

    - !KLN+(!K+L+N)K!L!M!N  Identity Law

    - !KLN + K!L!M!N!K + K!L!M!NL+K!L!M!NN  Distribution

    - !KLN + 0 + K!L!M!NL+K!L!M!NN  Complement Law

    - !KLN + K!L!M!NL+K!L!M!NN  Identity Law

    - !KLN + 0 +K!L!M!NN  Complement Law

    - !KLN +K!L!M!NN  Identity Law

    - !KLN + 0  Complement Law

    - !KLN  Identity Law

    - Simplifies down to !K*L*N

Therefore, there is only one solution:

K = 0, L = 1, N = 1

- Q = !X*!Y*Z + X*Y!Z + !X*Y*!Z + X*!Y*!Z

| X |  | Y | Z | !X*!Y*Z | X*Y*!Z | !X*Y*!Z | X*!Y*!Z | Q |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 |  | 0 | 1 | 1 | 0 | 0 | 0 | 1 |
| 0 |  | 1 | 0 | 0 | 0 | 1 | 0 | 1 |
| 0 |  | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| 1 |  | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 1 |  | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 1 |  | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 1 |  | 1 | 1 | 0 | 0 | 0 | 0 | 0 |

  - K-map:

  - Simplifying

!X!YZ+Y!Z(X+!X)+X!Y!Z  Distributive Law

!X!YZ+Y!Z*1+X!Y!Z  Complement Law

!X!YZ+Y!Z+X!Y!Z  Identity Law

!X!YZ+!Z(X!Y+Y)  Distributive Law

!X!YZ+!Z(X+Y)  Absorption Law

!X!YZ+!ZX+!ZY  Distribution

Answer: !XYZ+!ZX+!ZY

  - !A!B!C+ !AB!C+!ABC+ABC

  - !A!C(!B+B)+!ABC+ABC  Distributive Law

  - !A!C*1+!ABC+ABC  Complement Law

  - !A!C+!ABC+ABC  Identity Law

  - !A(BC+!C)+ABC  Distributive Law

  - !A(B+!C)+ABC  Absorption Law

  - !AB+!A!C+ABC  Distribution

  - !A!C + B(AC + !A)  Distributive Law

  - !A!C+B(C+!A)  Absorption Law

  - !A!C+BC+B!A  Distribution

  - BC + !C!A  Consensus

  - (B+!C)(C+!A) / BC + !C!A

Answer: BC + !C!A

- (Submitted on Submitty hw04_SOP.circuit)

  - Using OR only using NOR Gates (next page):

  - Using AND only using NOR Gates:

  - Using NOT only using NOR Gates:

- 10.13.1

  - 5ED4 in binary: 0101 1110 1101 0100

  - 07A4 in binary: 0000 0111 1010 0100

  - 5ED4 – 07A4 = 0101 1110 1101 0100 - 0000 0111 1010 0100

  - = 0101 0111 0011 0000  5730 in hexadecimal format

Answer: 5730 (hexadecimal)

- 10.13.2

  - Neither of the hexadecimal values are negative so we calculate it

| 5ED4 | 0101 | 1110 | 1101 | 0100 |
| --- | --- | --- | --- | --- |
| 07A4 | 0000 | 0111 | 1010 | 0100 |
| 5ED4 – 07A4 | 0101 | 0111 | 0001 | 0000 |
|  | 5 | 7 | 3 | 0 |

- 10.13.6

  - 185 = 10111001

  - 122 = 01111010

  - Add 1 to complement of 122 = 10000110 (2’s complement of 122)

| Binary value of 185 | 10111001 |
| --- | --- |
| Complement of 122 (2’s) | 10000110 |
| 185+2’s complement of 122 | 100111111 overflow |

Answer: -193

185 – 122 = -193

- 10.13.10

  - 151 & 214 both are stored in 2’s complement form

  - 151 & 214 in 2’s complement form

    - 151 = 10010111  01101001 (2’s complement form)  105

    - 214 = 11010110  00101010 (2’s complement form)  42

  - So, 151-214 = 105 – 42 = 63

Answer: 63

- 10.13.11

  - 151 & 214 both are stored in 2’s complement form

  - 151 & 214 in 2’s complement form

    - 151 = 10010111  01101001 (2’s complement form)  105

    - 214 = 11010110  00101010 (2’s complement form)  42

  - So, 151-214 = 105 + 42 = 147

Answer: 147

- 10.13.20

  - 0x0C000000 = 0000 1100 0000 0000 0000 0000 0000 0000

  - 0*167 + 12*166+ 0*165+ 0*164+ 0*163+ 0*162+ 0*161+ 0*160

  - 0 + 12 + 16777216 + 0 + 0 + 0 + 0 + 0 + 0

  - = 201326592

Answer: 201326592

- 10.13.21

  - 0x0C000000 = 0000 1100 0000 0000 0000 0000 0000 0000

  - In Binary 0C000000 = 00001100000000000000000000000000

  - 00001100000000000000000000000000

  - 000011 00000000000000000000000000

  - 000011  I-type instruction  000011 is jal

  - therefore, it turns into jal 0

Answer: jal 0

- Give a reason why we use two’s complement representation for negative numbers in computer arithmetic. Give and example of its usage.

  - 2’s complement is 1’s complement + 1

  - 2’s complement system you can use the same logics/circuits on binary numbers for addition and subtraction. It makes the system easier to implement and have easier precision arithmetic. It also excludes any negative zero cases.

  - For example, adding 6+3 using 5 2 but complement representation

  - 6  00110, 3  00011

  - 00110 + 00011 = 01001 (9), 6+3 = 9

  - So, this is why we use 2’s complement.